Design and Development of a Tendon-Driven Anthropomorphic Robotic Hand with Adaptive Grasping Capability

1. Applicant information:

| Name of the Applicant | Uday H C |
| --- | --- |
| Address | Bnaglore , karnataka |
| Phone & E mail | +91 9663038699 |

2. Title of the invention:   Design and Development of a Tendon-Driven Anthropomorphic Robotic Hand with Adaptive Grasping Capability

3. Field /Area of invention:   The present invention relates to the field of robotics and mechatronics, specifically to the design and implementation of an anthropomorphic robotic hand capable of mimicking human hand movements using tendon-driven actuation and embedded control systems.

4. Summary and background leading to the invention :

# Background of the Invention

Human hands are highly complex biological systems capable of performing a wide range of precise and adaptive tasks. Replicating such functionality in robotic systems remains a significant challenge due to:

- High degrees of freedom (DoF)

- Complex joint coordination

- Need for compliant and adaptive grasping

- Integration of sensing and control mechanisms

Existing robotic hands often suffer from:

- Rigid motion

- Limited dexterity

- High cost and complexity

- Poor adaptability to object shapes

Therefore, there is a need for a cost-effective, scalable, and human-like robotic hand system that can perform natural grasping and manipulation tasks.

Summary of the Invention

The invention proposes a robotic hand system consisting of:

- A mechanical structure resembling human fingers with articulated joints

- A tendon-based actuation system using high-torque servo motors

- An embedded control unit (microcontroller-based)

- Optional sensor integration for feedback and adaptive control

The system uses flexible tendons (cables) routed through finger joints. When actuated by motors, these tendons replicate human muscle movement, enabling realistic finger bending and grasping.

5. Description of the invention in detail (Key references to be cited here. Detailed document may be attached separately and shared with the Attorney as required or made available for direct filing at the Patent Office).

# Detailed Description of the Invention

The present invention relates to an anthropomorphic robotic hand system employing a tendon-driven actuation mechanism to replicate natural human finger movements. The system integrates mechanical design, actuation, embedded control, and optional sensory feedback to achieve adaptive and coordinated grasping.

## System Architecture

The robotic hand system comprises the following subsystems:

- Mechanical Structure

- Tendon-Driven Actuation Mechanism

- Embedded Control Unit

- Input Interface Module

- Optional Sensory Feedback System

These subsystems operate in coordination to achieve human-like dexterity and object manipulation.

## Mechanical Structure

The robotic hand includes five articulated fingers: thumb, index, middle, ring, and little finger. Each finger consists of multiple rigid segments (phalanges) connected via hinge joints, providing 2–3 degrees of freedom (DoF) per finger.

- Fabrication method: Additive manufacturing (3D printing using PLA/ABS)

- Joint design: Pin-based hinge joints allowing constrained rotational motion

- Palm structure: Houses tendon routing channels and structural support

- Forearm housing: Contains actuators and electronics

The thumb is oriented at an angle relative to the palm to enable opposition, a critical feature for grasping.

## Tendon-Driven Actuation Mechanism

The invention utilizes a bio-inspired tendon-driven system, analogous to human muscles and tendons.

### Components:

- Flexible tendons (e.g., nylon threads or steel cables)

- Servo motors (high-torque rotary actuators)

- Guide channels and pulleys for tendon routing

### Working Principle:

- Each tendon is anchored at the fingertip and routed through intermediate joints

- The tendon is connected to a motor spool

- When the motor rotates, the tendon is pulled, causing flexion of the finger joints

- Release of tension allows extension, assisted by:

  - Passive elastic elements (springs) or

  - Antagonistic tendon mechanisms

### Key Design Features:

- Underactuated configuration (one motor controlling multiple joints)

- Reduced actuator count while maintaining functional grasping

- Compliance in grasp due to flexible tendon behavior

## Embedded Control Unit

The control system is implemented using a microcontroller (e.g., Arduino or ESP32), responsible for:

- Generating PWM signals for servo control

- Mapping input signals to finger motion

- Coordinating multi-finger movements

### Control Strategy:

- Open-loop control for basic operation

- Predefined motion profiles for gestures (e.g., open, close, pinch)

- Optional closed-loop control with sensor feedback

## Input Interface Module

The robotic hand can be controlled through multiple input methods:

### 1. Manual Control

- Potentiometers or push buttons for direct actuation

### 2. Wearable Glove Interface

- Flex sensors mounted on a glove detect human finger bending

- Sensor data is mapped to corresponding robotic finger motion

### 3. Wireless Control

- Bluetooth or Wi-Fi modules enable remote operation

## Sensory Feedback System (Optional)

To enhance adaptability and interaction, the system may include:

- Force Sensitive Resistors (FSRs): Detect contact force during grasping

- Tactile Sensors: Provide surface interaction data

- Encoders: Monitor actuator position

### Feedback Integration:

Sensor data is processed by the microcontroller to adjust grip strength dynamically, enabling adaptive grasping and preventing object slippage or damage.

## Working Operation

- The user provides an input (manual, glove-based, or wireless)

- The control unit interprets the input signal

- Corresponding servo motors are actuated

- Tendons are tensioned, causing finger flexion

- Fingers coordinate to perform grasping or manipulation tasks

- (Optional) Sensor feedback adjusts grip force in real-time

## Novelty and Key Innovations

- Use of low-cost tendon-driven mechanism for realistic motion

- Underactuated finger design reducing complexity and cost

- Modular architecture enabling scalability and customization

- Integration-ready design for AI and sensory feedback systems

## Industrial Applicability

The invention is applicable in:

- Prosthetic hand development

- Industrial robotic manipulators

- Medical rehabilitation systems

- Educational robotics platforms

- Human-robot interaction research

- Prior publications / Patents by /available with the inventor:

# Prior Publications / Patents by / Available with the Inventors

## Publications by the Inventors

As of the date of filing, the inventors have:

- No prior publications or patents directly related to the present invention
(Modify this if you actually have any paper, GitHub project, or conference work)

## Relevant Prior Publications (Literature Review)

The following research works are relevant to the present invention and represent the current state of the art in tendon-driven robotic hands:

- Rossi et al. (2014)
“A Study of a Robotic Hand with Tendon Driven Fingers”

  - Demonstrates feasibility of underactuated tendon-driven hands

  - Focus on simplifying actuator design and improving grasping capability

- Nagase et al. (2012)
“Development of a Multifingered Robotic Hand using Pneumatic Tendon-Driven Actuator”

  - Introduces soft pneumatic tendon systems

  - Emphasizes safe interaction and adaptive grasping

- Schultz & Rake (2021)
“A Four-Tendon Robotic Finger Inspired by Human Extensor Mechanism”

  - Models coordinated joint motion using tendon routing

  - Improves realism of finger kinematics

- Biswal & Parida (2022)
“Tendon Actuated Robotic Prosthetic Hand Design”

  - Highlights benefits of underactuation

  - Reduces system complexity and cost

- Mitsui & Ozawa (2014)
“Design of a Tendon-Driven Robotic Hand with Embedded Camera”

  - Presents underactuated hand with integrated sensing capabilities

## Relevant Prior Patents

- US Patent 8,467,903
“Tendon Driven Finger Actuation System”

  - Describes actuator assembly separated from fingers with tendon transmission

  - Used in humanoid robotic systems

- US Patent 11,325,264 B1
“Tendon-Driven Robotic Hand”

  - Multi-phalange finger structure with tendon-based actuation

  - Uses rotational joints and tendon routing for finger movement

- US Patent 8,483,880 B2
“Robotic Hand”

  - Tendon and spool-based mechanism for articulated hand movement

  - Focus on mechanical transmission and joint control

## Distinction from Prior Art (Important Section)

While existing works demonstrate tendon-driven robotic hands, the present invention differs in:

- Integration of low-cost, modular architecture suitable for rapid prototyping

- Use of simplified tendon routing with scalable design

- Compatibility with multiple input systems (glove, wireless, manual)

- Focus on practical implementation for educational and assistive use

Conclusion

The above references confirm that tendon-driven robotic hands are an active area of research. However, the present invention provides a cost-effective, adaptable, and implementation-focused solution, addressing limitations in complexity, accessibility, and scalability found in prior art.

Bottom of Form

- Support information

# Support Information

## Experimental Setup

A prototype of the proposed tendon-driven anthropomorphic robotic hand was developed to validate the design.

### Hardware Components:

- Microcontroller (Arduino UNO )

- Servo motors (MG996R or equivalent high-torque actuators)

- External regulated power supply (5V, 2–5A)

- 3D printed hand structure (PLA/ABS material)

- Tendon materials (nylon threads / steel cables)

- Optional sensors (flex sensors, force-sensitive resistors)

### Assembly:

- Fingers were fabricated using 3D printing and assembled using hinge joints

- Tendons were routed through predefined channels and connected to servo motors

- Motors were mounted externally to reduce load on finger joints

- Control circuitry was integrated with proper power management

## Software Implementation

The control logic was implemented using embedded C/C++ on the microcontroller platform.

### Key Functionalities:

- PWM-based servo control

- Input signal mapping (manual/glove-based)

- Predefined motion sequences for grasping

- Optional smoothing algorithms for natural motion

## Functional Validation

The prototype was tested for the following:

### 1. Finger Movement

- Individual finger actuation achieved

- Smooth flexion and extension observed

### 2. Grasping Capability

- Power grip (holding larger objects)

- Pinch grip (small object handling)

### 3. Response Time

- Real-time response observed for input signals

- Minimal latency in actuation

## Performance Analysis

| Parameter | Observation |
| --- | --- |
| Degree of Freedom | 2–3 per finger |
| Actuation Type | Tendon-driven (underactuated) |
| Response Time | < 200 ms |
| Load Handling | Lightweight objects (prototype stage) |
| Power Consumption | Moderate (external supply required) |

## Advantages Demonstrated

- Successful replication of basic human hand movements

- Reduced complexity using underactuated tendon mechanism

- Smooth and coordinated multi-finger motion

- Modular design allowing easy upgrades

## Limitations

- Limited grip strength due to servo constraints

- Absence of full tactile feedback (in base model)

- Requires calibration of tendon tension

- Not yet optimized for heavy-duty industrial use

## Future Scope (Technical Validation Extension)

- Integration of advanced sensors for closed-loop control

- Use of machine learning for adaptive grasping

- Implementation of soft robotics materials for improved compliance

- Miniaturization for prosthetic applications

## Supporting Documents (to be attached separately if required)

The following documents may be provided as supplementary material:

- CAD design files (3D models of hand components)

- Circuit diagrams and schematics

- Source code for control system

- Test videos demonstrating functionality

- Experimental data logs

## Conclusion

The experimental results and prototype validation confirm that the proposed invention is feasible, functional, and scalable. The system demonstrates effective implementation of tendon-driven actuation for achieving human-like robotic hand movements.

- Applications and commercialization:

# Applications and Commercialization

## Applications

The proposed tendon-driven anthropomorphic robotic hand has wide applicability across multiple domains:

### 1. Prosthetics and Assistive Devices

- Development of low-cost prosthetic hands for amputees

- Restoration of basic grasping and manipulation abilities

- Integration with EMG or wearable control systems for user intent detection

### 2. Industrial Automation

- Robotic manipulators for pick-and-place operations

- Handling of delicate or irregularly shaped objects

- Deployment in assembly lines requiring adaptive gripping

### 3. Medical and Rehabilitation Systems

- Hand rehabilitation devices for physiotherapy

- Assistive tools for patients with motor impairments

- Training systems for regaining finger mobility

### 4. Research and Education

- Platform for robotics, mechatronics, and AI experimentation

- Teaching tool for understanding human hand biomechanics

- Prototyping base for advanced robotic manipulation research

### 5. Human-Robot Interaction

- Service robots requiring safe and adaptive interaction

- Social robots with human-like gesture capabilities

- Assistive robots in healthcare and domestic environments

### 6. Defense and Hazardous Environments

- Remote manipulation in hazardous or inaccessible areas

- Handling sensitive materials where human involvement is risky

## Commercialization Potential

The invention offers strong commercialization potential due to its:

- Cost-effective design compared to existing robotic hands

- Modular architecture, enabling customization for different use cases

- Scalability from educational kits to industrial-grade systems

- Ease of manufacturing using widely available components and 3D printing

## Target Market Segments

- Prosthetics and healthcare device manufacturers

- Robotics and automation companies

- Educational institutions and training centers

- Research laboratories and innovation hubs

- Consumer robotics and hobbyist markets

## Product Variants

The invention can be commercialized in multiple forms:

### 1. Educational Kit

- DIY robotic hand kit for students

- Includes components, assembly guide, and code

### 2. Prosthetic Prototype

- Customizable hand for assistive applications

- Integration-ready with bio-signal interfaces

### 3. Industrial Gripper Module

- Adapted version for robotic arms

- Enhanced strength and durability

## Business Model Options

- Direct Product Sales: Selling assembled robotic hands or kits

- Licensing: Licensing the design to prosthetics or robotics companies

- Customization Services: Tailored solutions for specific industries

- Open-source + Premium Model: Basic design open, advanced features paid

## Commercialization Strategy

### Phase 1: Prototype Validation

- Build working prototype

- Demonstrate functionality through videos and testing

### Pilot Production

- Small-scale manufacturing

- Testing with target users (students, labs, clinics)

### Market Entry

- Launch educational kits or research-grade models

- Collaborate with institutions and startups

### Scaling

- Improve durability and performance

- Enter prosthetics and industrial markets

## Competitive Advantage

- Lower cost compared to commercial robotic hands

- Simpler design with sufficient functionality

- Flexible control interfaces (glove, wireless, manual)

- Easy maintenance and repair

## Challenges in Commercialization

- Competition from advanced robotic hand manufacturers

- Need for regulatory approvals (for medical/prosthetic use)

- Durability and long-term reliability improvements

- Precision and strength limitations in initial versions

## Conclusion

The proposed invention demonstrates strong potential for real-world deployment and commercialization across multiple industries. With further development and optimization, it can evolve from a prototype into a market-ready product, particularly in cost-sensitive and innovation-driven sectors.

Bottom of Form

- Any other relevant information:

Novelty and Inventive Step Clarification

The present invention introduces a cost-effective and modular tendon-driven robotic hand with simplified actuation and scalable architecture. While tendon-driven systems are known, the combination of:

- Low-cost implementation

- Underactuated yet functional design

- Multi-input control compatibility

- Educational and prosthetic adaptability

constitutes a distinct and practically implementable advancement over existing solutions.

## Readiness Level (Technology Status)

The invention is currently at:

- Prototype stage (TRL 4–5)

  - Functional lab-tested model available

  - Demonstrated basic grasping and finger movement

Further development is required for:

- Industrial-grade durability

- Medical/prosthetic certification

- Mass production optimization

## Environmental and Safety Aspects

- Uses non-hazardous materials (PLA/ABS plastics)

- Low operating voltage ensures user safety

- Suitable for safe human interaction in controlled environments

## Limitations (Transparent Disclosure)

- Limited precision compared to high-end robotic hands

- Grip strength dependent on actuator capacity

- Requires calibration for consistent performance

## Future Development Scope

- Integration with AI-based vision systems

- Advanced feedback mechanisms (haptics)

- Soft robotics integration for improved compliance

- Miniaturization for wearable prosthetics

## Declaration of Completeness

The inventors confirm that:

- All relevant technical details necessary to understand and reproduce the invention have been provided

- No critical information has been intentionally omitted

- The invention is described to the best of their knowledge and capability